# Tidal assist: 3 knot paddling speed

| Tidal assist | Timesave |
| --- | --- |
| 0.5 | 15% |
| 1 | 25% |
| 1.5 | 33% |
| 2 | 40% |
| 2.5 | 45% |
| 3 | 50% |
| 3.5 to 4 | 55% |
| 4.5 to 5 | 60% |
| 5.5 to 6 | 66% |

# Tidal retard: 3 knot paddling speed

| Tidal retard | Time increase |
| --- | --- |
| 0.5 | 20% |
| 1 | 50% |
| 1.5 | 100% |
| 2 | 200% |
| 2.5 | 500% |
| 3 | No movement |

## Trip plan

| Notify somebody of your plan (safety reasons) |  |
| --- | --- |
| Food + hot drink/water |  |
| Notify somebody of safe return |  |

# Night paddling PREPARATION!

Well dressed + fed

Crucial bearings ready

Method for checking all present

(1,2,3,4,5 etc.)

Group formation with lightsticks

Mark sectors of lights on map

Mark lights on map

One long blast means “gather”

# Group

Experience?

Medical issues?

Kit?

Quick reckoning

NM to minutes (if speed 3knots): times by ten then double it (so 2.3nm =23=46mins (followed by tidal assist/retard calc)

Wave height: F4=1m F5=2M F6=3M.

50/90 rule: flow 50/90/max/90/50.

Twelfths rule: height 123321.

Rule of thirds: speed 1/3 2/3 3/3 3/3 2/3 1/3

Knots to Beaufort: (knots/5)+1=BFT.

Beaufort to knots: (BFT-1)*5=knots

Wind retard (using Beaufort) 0=3knots, 1=3knots, 2=2.5knots, 3=2knots, 4=1.5knots, 5=0.5knots, 6= stay still

| Force+description | Speed | Description | Waves |
| --- | --- | --- | --- |
|  | knots |  | metres |
| 0 | 0-1 | calm | 0 |
| 1 smooth | 1-3 | light air | 0 |
| 2 smooth | 4-6 | light breeze | 0.1/0.3 |
| 3 slight | 7-10 | gentle breeze | 0.4/1 |
| 4 slight | 11-16 | moderate breeze | 1/1.5 |
| 5 rough | 17-21 | fresh breeze | 2/2.5 |
| 6 rough | 22-27 | strong breeze | 3/4 |
| 7 very rough | 28-33 | near gale | 4/5.5 |
| 8 very rough | 34-40 | gale | 5/7.5 |
| 9 very high | 41-47 | severe gale | 7/10 |
| 10 very high | 48-55 | storm | 9/12.5 |
| 11 very high | 56-63 | violent storm | 11/16 |
| 12 phenomenal | 64+ | hurricane | 14+ |

# Lights

(F) Fixed continuous

(FL) Flashing

(GP FL) Flashing groups

(V QK FL) Very quick flash

(Occ) Light longer than dark

(GP Occ) Group occulting

(Iso) phase equal light dark

Created by Julian Patrick - 2006